Journal of Geophysical Research: Atmospheres

Supporting Information for

The importance of NOx control for peak ozone mitigation based on a sensitivity study using CMAQ-HDDM-3D model during a typical episode over the Yangtze River Delta region, China

Yangjun Wang1,2†, Elly Arukulem Yaluk 1,2†, Hui Chen1,2, Sen Jiang1,2, Ling Huang1,2, Ansheng Zhu1,2, Shilin Xiao1,2, Jin Xue1,2, Guibin Lu3, Jinting Bian1,2, Manomaiphiboon Kasemsan4,5, Kun Zhang1,2, Hanqing Liu1,2, Huanhuan Tong1,2, Chel Gee Ooi6, Andy Chan7, Li Li 1,2*

1School of Environmental and Chemical Engineering, Shanghai University, Shanghai 200444, China

2 Key Laboratory of Organic Compound Pollution Control Engineering, Shanghai University, Shanghai 200444, China

3 School of Economics, Shanghai University, Shanghai, 200444, China

4 The Joint Graduate School of Energy and Environment, King Mongkut’s University of Technology, Thonburi, Bangkok 10140, Thailand

5 Center of Excellence on Energy Technology and Environment, Ministry of Higher Education, Science, Research and Innovation, Bangkok, 10140, Thailand

6 Institute of Climate Change (IPI), National University of Malaysia (UKM), 43600, Bangi, Selangor, Malaysia

7 Department of Civil Engineering, University of Nottingham Malaysia, Semenyih 43500, Selangor, Malaysia

† These authors contributed equally to this study.

*Corresponding author: School of Environmental and Chemical Engineering, Shanghai University, Shanghai, 200444, China; Tel:+8618019005281 ; E-mail (lily@shu.edu.cn).

Contents of this file

Text S1

Text S2

Figures S1 to S16

Tables S1 to S2

Text S1. Model Performance Evaluation

In this study, the simulation performance of the WRF and CMAQ models, as well as the sensitivity performance of HDDM (see section 3.5 in the main text) was thoroughly evaluated. However, due to limited meteorological observations, the WRF model performance was assessed for the period from 17th to 31st July 2018, which adequately covers the main focus period of this work. We evaluated WRF simulations against the observations of air temperature at 2m (T2), wind speed at 10m (WS10) and relative humidity (RH) in 41 major cities within YRD. The meteorological observations were obtained at specific monitoring stations in each city, while the model simulations are the grid-cell (4km×4km) average values corresponding to each of the monitoring stations. The statistical metrics used in the meteorology evaluation include the mean bias (MB) and normalized MB (NMB); normalized mean error (NME), root mean square error (RMSE) and the correlation coefficient (R) (Equations  to ).

Generally, positive values of MB and NMB indicate a model prediction that exceeds the observation, whereas negative values indicate an underprediction. For R, a coefficient of 1 indicates a perfect linear relationship between the model predictions and ground-based measurements, whereas a correlation coefficient of 0 statistically indicates no linear relationship. Because IAO reflects the degree to which the observations are accurately assessed by model predictions, an IOA of 0.0 typically signify an inaccurate prediction and a value of 1.0 indicates perfect accuracy between the predicted and observed values. IOA and R values above 0.5 are commonly considered to represent good predictions.

The simulated hourly variations of T2, WS10 and RH compared well with ground-level measurements over the YRD region during the study period (Figure S1), generally elucidating the relatively stagnant, sunny, and dry atmospheric conditions (after the typhoon around 22-23 July). These conditions are favorable for a high photochemical activity. Figure S2 shows box plots for statistical comparison of hourly simulated and observed values of T2, WS10 and RH averaged for the 41 cities in the YRD region. It is obvious that the model captured well the hourly variations of T2, WS10, and RH all with R values above 0.90, which adequately represent good predictions reflecting the observed summertime weather conditions over the YRD region during the time period from 17th to 31st July, 2018.

The WRF model revealed a conspicuously low bias in predicting WS10 (NMB = -21.82%) particularly in the cities with low O3 concentrations (such as -50.5% of Anqing and -65.1% of Huangshan), which were not the main focus cities or areas in this study. Nonetheless, this underestimation is generally acceptable because of the small RMSE value (0.85) and high R value (0.90). Similarly, even with the slight underestimation of RH (NMB = -5.9%), the model achieved high R (0.95) and low RMSE values (7.15%), which generally shows that the WRF simulation of RH over the YRD is reasonable. For T2, the small magnitudes of MB (0.3°C), NMB (1.01%), NME (3.45%), RMSE (1.26°C), and the high R (0.93) value, all highlight a good agreement between the simulation results and observations. Based on the above statistical results, the meteorological simulations for this study are comparable with previous studies [Huang et al., 2021; Shi et al., 2021; Shu et al., 2020; X Wang et al., 2021; Zhan et al., 2020]. More details of the evaluation metrics for meteorology model performance are shown in Table S1. Overall, the WRF model reasonably captured the prevailing weather conditions during the study period and provide reliable meteorological inputs for the CMAQ model.

For a thorough evaluation of the air quality model (AQM) performance, we compared the CMAQ simulation results against ground-level measurements for O3 and NO2 between 17th July to 12th August, 2018. We then evaluated the hourly and the daily-average concentrations for each monitoring site in each of the 41 cities. This necessitated a robust model performance evaluation for all the major cities in the YRD region, with specific highlights in the key representative (provincial capital) cities. Besides the statistical metrics used in the meteorology evaluation, the index of Agreement (IAO, Eq. ) was included in the evaluation of CMAQ model performance.

By comparing the hourly simulations and observations, the NMB (R, IOA) values of O3 were -32% (0.73, 0.80), 3% (0.8, 0.89), -1% (0.79, 0.89) and 15% (0.75, 0.85) for the four representative cities of Shanghai, Hangzhou, Nanjing and Hefei, respectively. For the NO2 simulation, the model achieved NMB (R, IOA) values of -35% (0.58, 0.64), -20% (0.62, 0.72), -26% (0.68, 0.77) and -18% (0.64, 0.76), respectively, in Shanghai, Hangzhou, Nanjing and Hefei as shown in Figure S3. The observations for each city are the average values of the observed data in all the national controlled observation stations in that city, while the simulations for a city represent the average of the simulated data in all grid-cells corresponding to these national controlled monitoring stations in the city. Actually, the concentration in a specific station usually has discrepancy with the spatial average concentration in the corresponding grid-cell over the station. Sometimes this kind of discrepancy is significant, especially when there is a remarkable spatial variability. Compared with O3, the regionality of NO2 is relatively weak. This means that the spatial variability of NO2 is stronger, and so the kind of spatial discrepancy for NO2 would be relatively larger than O3.

On that note, as shown in Figure S4, NO2 was generally underestimated during the daytime, mainly due to a probable underestimation of NOx emission or meteorological simulation discrepancy, such as overestimated PBL. This underestimation of the hourly NO2 simulation is mainly reflected in the region with low concentrations of NO2 (Figure S5). As shown in both Figure S5 and Figure S6, the CMAQ model had a better ability to simulate O3. Figure S7 shows the timeseries comparisons between observations and simulation of O3 and NO2 concentrations in the four representative cities. We noted a significant underestimation of NO2 in Shanghai around midnight of 26th July until the early morning hours on 27th July, which corresponded with a significant underestimation of O3 during the daytime of 27th July. Similar phenomenon occurred in Nanjing around the same time. These hourly values of simulated NO2 concentration with relatively large deviations are also clearly depicted in Figure S3. It is important to note that this sudden increase in NOx emissions during the stated time period above was not accounted by the emissions inventory, and hence the model simulated the “normal” NOx levels. In the general comparison between the simulation results and observations of NO2 and that of O3, we noted that the simulated values of the latter are largely underestimated, which further supported the findings of this study that: NOx emission controls can (in a general context) reduce O3 concentration over the YRD region. Overall, both NO2 and O3 simulations captured well the variations of the observed values during the study period.

The statistical metrics for model performance evaluation for the entire region and for each of the 41 cities in YRD are summarized in Table S2,  Table S3 (for O3) and Table S4 (for NO2). Although the model generally underpredicted O3 over the region (NMB about -11%), the average R and IOA for O3 simulation at all the monitoring stations across the YRD region was about 0.72 and 0.81 (Table S2), respectively, which represent good predictions, and is within the recommended model performance criteria suggested by [Emery et al., 2017]. Moreover, O3 simulation attained R>0.7 in 28 cities, and R of between 0.56-0.69 in another 11 cities (Table S3). With this, about 24 cities satisfied the NMB criteria for O3 benchmark (±15%) as suggested by Emery et al. (2017). Nevertheless, the model performance of several other cities was generally poor (particularly for the coastal cities), which could be mainly due to the influence of sea breeze that is biased in the simulation, or the background concentration levels. Most of the cities with underpredicted O3 exhibited very low pollution concentrations during the study period, such as Chizhou, Jinhua, Lu’an, Lianyungang, and Huangshan (Table S3). That said, these cities with poor simulation performance were not the main focus of this study because they generally have good air quality. The provincial capital cities and their surrounding areas are the focus of this study, and are also the main areas of interest by the government due to their heavy ozone pollution, high economic status, and high population density.

Overall, the CMAQ model performance in this study (particularly for O3 simulation) is commendable and is comparable with previous studies [Hu et al., 2018; Liu et al., 2020; H Wang et al., 2022; X Wang et al., 2021]. Ultimately, since there was a general concordance between the model and ground-based observations of O3 at both spatial and temporal scales, the CMAQ model results based on the above evaluation satisfies the general objective of this study for a detailed analysis O3-NOx-VOC sensitivity for the YRD region.

Text S2. Process analysis applied at a regional scale

From a spatial perspective, the positive contribution of vertical diffusion (VDIF) and gas-phase chemistry (CHEM) to O3 was significant over the YRD region during the modeled episode from 24th to 31st July, 2018 (Figure S11). Conspicuously, the contributions of CHEM occur more significantly in the central and southeastern areas (Figure S11b), while VDIF contributions appear more prominently in the northern and eastern areas (Figure S11c). The high positive CHEM contributions are generally linked to the relative abundance of O3 precursors, unless the ozone titration rate due to large NO emissions is much higher than the ozone generation rate. With regards to DDEP, it is the main sink for O3 over the modeling domain exhibiting high values in the northeast inland areas, whereas lesser values are observed over water bodies and around highly urbanized locations (Figure S11d). Understandably, this is because less air pollution occurs over oceans, lakes, and rivers, while in the urban cores O3 is primarily consumed via NO titration effect. In general, the contribution of CHEM is significant in southern Jiangsu, Hangzhou Bay, and coastal areas, while the contribution of DDEP in central Jiangsu, southeastern and north-central Anhui is relatively large. It is worth noting that significant CHEM's contribution also appears over Hangzhou Bay, and the sea nearer to the land, usually due to abundant O3 precursors transported from nearby areas and very little NO to destruct O3.

Figure S1. Hourly variation of observed (red line) and simulated (blue line) values of (A) T2, (B) WS10 and (C) RH averaged on all 41 cities in the YRD region. The shaded area represents the variation range of hourly observations across the 41 cities (spatial variability).

Figure S2. Box plots for evaluation metrics based on the comparison of hourly simulated and observed values T2, WS10 and RH for 41 cities in the YRD region. (A) mean bias (MB), (B) normalized MB (NMB), (C) normalized mean error (NME), (D) root mean square error (RMSE) and (E) the correlation coefficient (R).

Figure S3. Scatter plots of simulated and observed hourly concentrations of (A) O3 and (B) NO2 for all sites in Shanghai (black dots), Hangzhou (red dots), Nanjing (blue dots) and Hefei (green dots). Each dot represents hourly value averaged for all monitoring stations (and the corresponding grid cell average) in each city.

Figure S4. Hourly variations of (A) O3 and (B) NO2 concentrations compared between ground-based observations (blue line) and model simulations (purple line) averaged by all the (223) national controlled monitoring stations in 41 cities across the YRD region.

Figure S5. The scattered plots comparing simulated and observed hourly concentrations of (A) O3 and (B) NO2 for the 41 cities over the YRD region. The values are averaged at all monitoring stations and the corresponding grid cell average in each city. Each dot represents bins, and the color of each bin denotes the number of points over the bin as shown in the color bar. The red line shows the linear regression line for all points over the drawing layout, and the dotted lines show ±50% deviation from the 1:1 black solid line.

Figure S6. Scatter plots comparing the daily concentrations of simulated and observed (A) O3 and (B) NO2 for the 41 cities over the YRD region. The values are averaged for each day at all monitoring stations and the corresponding grid cell average for each city. Each dot represents bins, and the color of each bin denotes the number of points over the bin as shown in the color bar. The red line shows the linear regression line for all points over the drawing layout, and the dotted lines show ±50% deviation from the 1:1 black solid line.

Figure S7. Timeseries comparisons between observations and simulation of O3 and NO2 concentrations between 17th July to 1st August 2018 in the key representative cities of (A) Shanghai, (B) Hangzhou, (C) Nanjing and (D) Hefei in the YRD region. The values are averaged across all the monitoring sites within each city.

Figure S8. Hourly variation of ground-based observations of O3 in typical cities of YRD from 17th July to 3rd August 2018. The cities are grouped into four distinct categories: (a) cities in the eastern coastal region, including Shanghai, Ningbo, Nantong, Shaoxing and Jiaxing; (b) the cities in southeast inland region, including Hangzhou, Suzhou, Huzhou Wuxi, and Changzhou; (c) cities in the central inland region, including Nanjing, Chuzhou, Maanshan, Xuancheng and Wuhu; and (d) the cities in the northwest region of YRD domain, including Hefei, Tongling, Bengbu, Suzhou (AH), and Huaibei. The dashed line shows China’s NAAQS for hourly O3 limit (200 µg/m3).

Figure S9. Spatial distribution of simulated O3 overlaid with the surface wind fields at 14:00 LT on (a) 19 July before typhoon, (b) 22 July during typhoon, (c) 24 July after typhoon, and (d) 29 July the peak of O3 episode after typhoon. These dates represent the distinct evolution patterns of the modeled O3 during the over the YRD region.

Figure S10. Daytime (9:00-14:00 LT) averaged vertical profile of gas-phase chemistry (CHEM) contributions for Ox (Ox = O3+NO2) (solid lines) and O3 (dotted lines) between 24th – 31st July 2018 in the typical (A) urban and (B) suburban sites in Shanghai, Hangzhou, Nanjing, and Hefei. The approximate height of each modeling layer is defined in Table S5.

Figure S11. Spatial plots showing daytime average (9:00-14:00 LT) concentrations of (a) surface O3 and the main contributions from (b) gas-phase chemistry (CHEM), (c) vertical diffusion (VDIF), and (d) dry deposition (DDEP). The values are averaged during the episode period between 24th -31st July 2018.

Figure S12: Time series of simulated O3 (grey line and black dots) and its respective first-order sensitivity coefficients to NOx (red), AVOC (blue) and BVOC (green) at suburban sites in (a) Shanghai, (b) Hangzhou, (c) Nanjing and (d) Hefei during the modelling period (17 July -2 August, 2018).

Figure S13: Time series of simulated O3 (grey line and black dots) and its respective first-order sensitivity coefficients to NOx (red), AVOC (blue) and BVOC (green) at urban sites in (a) Shanghai, (b) Hangzhou, (c) Nanjing and (d) Hefei during the modelling period (17 July -2 August, 2018).

Figure S14: Diurnal profiles of simulated O3 concentration (black dotted line) and their corresponding first-order sensitivity coefficients to NOx (red), AVOCs (blue), BVOCs (green) and second-order sensitivity to NOx (gray) in urban sites for (a) Shanghai, (b) Hangzhou, (c) Nanjing and (d) Hefei. The values are averaged over the episode period between 24th-31st July 2018.

Figure S15. Distribution of O3 sensitivity regimes averaged between 9:00-16:00 LT during a severe O3 episode on 24th -31st July 2018 over the YRD region. The different colors correspond to the different sensitivity regimes: VOC-sensitive (red), NOx-sensitive (green), mixed sensitivity (yellow), NOx-titration (purple) and insensitive (blue) regimes.

Figure S16: Scatter plots for hourly O3 sensitivity coefficients to AVOCs (x-axis) and NOx (y-axis) between 7:00 to 18:00 LT at urban sites in (a) Shanghai (b) Hangzhou (c) Nanjing and (d) Hefei during severe O3 episode between 24th-31st July. The various categories of O3 sensitivity regimes are classified using red dotted lines (Insensitive, NOx-sensitive, mix-sensitivity, VOC-sensitive and NOx-titration regimes) accordingly.

Table S1. The statistical metrics for the comparison between ground-based observations and simulation of T2, WS10, and RH for all the 41 cities over the YRD region (MB is mean bias; NMB is normalized mean bias; NME is normalized mean error; RMSE is Root mean square error; and R is correlation coefficient).

| Variables | MB | NMB | NME (%) | RMSE | R |
| --- | --- | --- | --- | --- | --- |
| T2 (°C) | 0.30 | 1.01 | 3.45 | 1.26 | 0.93 |
| WS10 (m/s) | -0.66 | -21.82 | 23.60 | 0.85 | 0.90 |
| RH (%) | -5.90 | -7.66 | 8.13 | 7.15 | 0.95 |

Table S2. The statistical metrics for the comparison between ground-based observations and simulation of O3 and NO2 for all the 41 cities over the YRD region (MB is mean bias; NMB is normalized mean bias; NME is normalized mean error; RMSE is Root mean square error; R is correlation coefficient, and IOA is thee index of Agreement).

| Variable | MB | NMB | NME (%) | RMSE | R | IOA |
| --- | --- | --- | --- | --- | --- | --- |
| O3 (µg/m3) * | -10.05 | -11% | 38% | 37.68 | 0.72 | 0.81 |
| NO2 (µg/m3) ** | -5.59 | -25% | 55% | 14.65 | 0.47 | 0.61 |

Table S3. Model performance of O3 concentration in 41 cities of YRD from 17th July to 12th August 2018. (MB is mean bias; NMB is normalized mean bias; NME is normalized mean error; RMSE is Root mean square error; R is correlation coefficient; and IOA is Index of agreement). The results of the key representative (provincial capital) cities are highlighted in bold.

| City | MB (µg/m3) | NMB (%) | NME (%) | RMSE (µg/m3) | R | IOA |
| --- | --- | --- | --- | --- | --- | --- |
| Shanghai | -23.67 | -32% | 44% | 45.37 | 0.73 | 0.80 |
| Lishui | 5.72 | 9% | 43% | 32.24 | 0.66 | 0.77 |
| Bozhou | -16.38 | -20% | 34% | 33.06 | 0.74 | 0.82 |
| Lu’an | -24.06 | -27% | 37% | 39.04 | 0.66 | 0.73 |
| Nanjing | -0.46 | -1% | 33% | 35.59 | 0.79 | 0.89 |
| Nantong | -20.02 | -28% | 49% | 44.92 | 0.74 | 0.82 |
| Taizhou | -4.39 | -8% | 43% | 29.57 | 0.74 | 0.85 |
| Hefei | -12.31 | -15% | 35% | 35.44 | 0.75 | 0.85 |
| Jiaxing | -12.05 | -15% | 42% | 43.40 | 0.76 | 0.86 |
| Ningbo | -14.25 | -23% | 42% | 37.02 | 0.74 | 0.83 |
| Anqing | -1.72 | -2% | 34% | 34.53 | 0.69 | 0.83 |
| Xuancheng | 4.46 | 6% | 30% | 26.55 | 0.72 | 0.84 |
| Suzhou | -20.94 | -22% | 35% | 40.14 | 0.71 | 0.76 |
| Suqian | -14.82 | -17% | 41% | 42.17 | 0.63 | 0.77 |
| Changzhou | -20.56 | -23% | 32% | 37.85 | 0.86 | 0.88 |
| Xuzhou | -25.98 | -29% | 40% | 43.31 | 0.74 | 0.75 |
| Yangzhou | -9.31 | -11% | 38% | 41.24 | 0.77 | 0.87 |
| Wuxi | -9.36 | -12% | 35% | 36.76 | 0.83 | 0.90 |
| Hangzhou | 1.98 | 3% | 32% | 33.29 | 0.80 | 0.89 |
| Chizhou | -3.17 | -4% | 37% | 41.30 | 0.59 | 0.77 |
| Taizhou | -12.35 | -16% | 33% | 32.62 | 0.84 | 0.90 |
| Huaibei | -33.17 | -34% | 45% | 53.85 | 0.56 | 0.64 |
| Huainan | -26.91 | -27% | 35% | 40.85 | 0.75 | 0.79 |
| Huai’an | -17.69 | -21% | 39% | 39.05 | 0.69 | 0.79 |
| Wenzhou | 4.96 | 10% | 54% | 35.20 | 0.73 | 0.83 |
| Huzhou | -2.13 | -3% | 39% | 43.05 | 0.75 | 0.86 |
| Chuzhou | -13.26 | -15% | 36% | 41.07 | 0.73 | 0.82 |
| Yancheng | -13.58 | -17% | 32% | 32.42 | 0.77 | 0.85 |
| Shaoxing | 4.63 | 7% | 39% | 36.89 | 0.76 | 0.87 |
| Wuhu | -13.10 | -15% | 39% | 43.81 | 0.67 | 0.77 |
| Suzhou | -6.48 | -9% | 33% | 30.86 | 0.85 | 0.92 |
| Bengbu | -17.85 | -20% | 34% | 37.52 | 0.74 | 0.81 |
| Quzhou | 2.53 | 4% | 33% | 28.76 | 0.75 | 0.84 |
| Lianyungang | -16.57 | -26% | 59% | 48.16 | 0.20 | 0.52 |
| Jinhua | -3.88 | -5% | 30% | 31.24 | 0.83 | 0.88 |
| Tongling | 3.17 | 4% | 43% | 39.33 | 0.60 | 0.77 |
| Zhenjiang | -25.30 | -26% | 35% | 40.59 | 0.80 | 0.83 |
| Fuyang | -12.20 | -15% | 35% | 35.61 | 0.64 | 0.78 |
| Maanshan | -7.36 | -8% | 30% | 33.68 | 0.79 | 0.87 |
| Huangshan | 25.78 | 57% | 57% | 29.73 | 0.67 | 0.61 |
| Zhoushan | -20.50 | -38% | 57% | 34.88 | 0.59 | 0.62 |

Table S4. Model performance of NO2 concentration in 41 cities of YRD from 17th July to 12th August 2018. (MB is mean bias; NMB is normalized mean bias; NME is normalized mean error; RMSE is Root mean square error; R is correlation coefficient; and IOA is Index of agreement). The results of the key representative (provincial capital) cities are highlighted in bold.

| City | MB (µg/m3) | NMB (%) | NME (%) | RMSE (µg/m3) | R | IOA |
| --- | --- | --- | --- | --- | --- | --- |
| Shanghai | -8.93 | -35% | 41% | 15.85 | 0.58 | 0.64 |
| Lishui | -10.11 | -68% | 69% | 11.59 | 0.53 | 0.51 |
| Bozhou | 2.66 | 18% | 93% | 18.17 | 0.37 | 0.49 |
| Lu'an | -9.12 | -36% | 58% | 16.95 | 0.29 | 0.45 |
| Nanjing | -7.97 | -26% | 39% | 16.03 | 0.68 | 0.77 |
| Nantong | -3.07 | -14% | 48% | 14.65 | 0.61 | 0.77 |
| Taizhou | -4.45 | -31% | 53% | 9.18 | 0.43 | 0.62 |
| Hefei | -5.34 | -18% | 39% | 14.09 | 0.64 | 0.76 |
| Jiaxing | -3.39 | -17% | 49% | 12.40 | 0.50 | 0.68 |
| Ningbo | -5.39 | -27% | 39% | 11.46 | 0.61 | 0.74 |
| Anqing | -3.08 | -15% | 56% | 15.82 | 0.41 | 0.62 |
| Xuancheng | -9.29 | -52% | 61% | 12.47 | 0.41 | 0.54 |
| Suzhou | -13.93 | -51% | 61% | 18.36 | 0.53 | 0.53 |
| Suqian | -3.17 | -17% | 51% | 11.78 | 0.43 | 0.61 |
| Changzhou | -2.76 | -11% | 42% | 13.43 | 0.62 | 0.74 |
| Xuzhou | 1.31 | 5% | 44% | 13.73 | 0.57 | 0.74 |
| Yangzhou | -3.52 | -14% | 49% | 15.81 | 0.48 | 0.69 |
| Wuxi | -1.19 | -4% | 35% | 14.64 | 0.63 | 0.78 |
| Hangzhou | -4.34 | -20% | 39% | 10.13 | 0.62 | 0.72 |
| Chizhou | -13.80 | -56% | 68% | 18.55 | 0.06 | 0.37 |
| Taizhou | 0.62 | 3% | 52% | 12.47 | 0.66 | 0.75 |
| Huaibei | 10.20 | 61% | 80% | 18.72 | 0.53 | 0.52 |
| Huainan | 5.40 | 29% | 68% | 17.75 | 0.44 | 0.43 |
| Huai'an | -4.44 | -25% | 48% | 10.33 | 0.57 | 0.72 |
| Wenzhou | -18.58 | -67% | 69% | 21.98 | 0.43 | 0.50 |
| Huzhou | -11.83 | -46% | 53% | 17.80 | 0.58 | 0.64 |
| Chuzhou | -12.39 | -46% | 53% | 17.71 | 0.48 | 0.58 |
| Yancheng | 3.26 | 25% | 55% | 10.06 | 0.55 | 0.65 |
| Shaoxing | 0.05 | 0% | 51% | 8.96 | 0.52 | 0.71 |
| Wuhu | -12.36 | -51% | 58% | 17.27 | 0.41 | 0.54 |
| Suzhou | -7.50 | -27% | 37% | 13.66 | 0.64 | 0.74 |
| Bengbu | -6.29 | -23% | 58% | 19.54 | 0.36 | 0.60 |
| Quzhou | -9.47 | -42% | 62% | 17.22 | 0.34 | 0.53 |
| Lianyungang | -13.32 | -99% | 99% | 14.68 | 0.03 | 0.36 |
| Jinhua | -7.58 | -40% | 56% | 13.60 | 0.39 | 0.58 |
| Tongling | -12.74 | -40% | 58% | 22.99 | 0.34 | 0.57 |
| Zhenjiang | -6.64 | -31% | 53% | 14.03 | 0.52 | 0.66 |
| Fuyang | 1.72 | 9% | 50% | 12.00 | 0.50 | 0.64 |
| Maanshan | -4.25 | -19% | 36% | 10.98 | 0.58 | 0.72 |
| Huangshan | -8.56 | -75% | 75% | 9.04 | -0.08 | 0.25 |
| Zhoushan | -2.45 | -17% | 63% | 11.25 | 0.24 | 0.48 |

Table S5. Vertical model layers and the corresponding approximate height in meters (m) as used in the CMAQ simulations for this study

| CMAQ Layer | Sigma/eta level | Pressure | Approx. height (m) |
| --- | --- | --- | --- |
| 1 | 0.995 | 1008.44 | 40 |
| 2 | 0.998 | 1001.69 | 96 |
| 3 | 0.980 | 993.98 | 161 |
| 4 | 0.970 | 984.35 | 242 |
| 5 | 0.956 | 970.87 | 358 |
| 6 | 0.938 | 953.53 | 508 |
| 7 | 0.893 | 910.18 | 893 |
| 8 | 0.839 | 858.17 | 1375 |
| 9 | 0.777 | 798.44 | 1959 |
| 10 | 0.702 | 726.20 | 2715 |
| 11 | 0.582 | 610.61 | 4061 |
| 12 | 0.4 | 435.30 | 6565 |
| 13 | 0.2 | 242.65 | 10528 |
| 14 | 0 | 50 | 19275 |

References

Emery, C., Z. Liu, A. G. Russell, M. T. Odman, G. Yarwood, and N. Kumar (2017), Recommendations on statistics and benchmarks to assess photochemical model performance, Journal of the Air & Waste Management Association, 67(5), 582-598, doi:10.1080/10962247.2016.1265027.

Hu, J., Y. Li, T. Zhao, J. Liu, X. M. Hu, D. Liu, Y. Jiang, J. Xu, and L. Chang (2018), An important mechanism of regional O3 transport for summer smog over the Yangtze River Delta in eastern China, Atmos. Chem. Phys., 18(22), 16239-16251, doi:10.5194/acp-18-16239-2018.

Huang, L., Q. Wang, Y. Wang, C. Emery, A. Zhu, Y. Zhu, S. Yin, G. Yarwood, K. Zhang, and L. Li (2021), Simulation of secondary organic aerosol over the Yangtze River Delta region: The impacts from the emissions of intermediate volatility organic compounds and the SOA modeling framework, Atmospheric Environment, 246, 118079, doi:10.1016/j.atmosenv.2020.118079.

Liu, T., et al. (2020), Driving Forces of Changes in Air Quality during the COVID-19 Lockdown Period in the Yangtze River Delta Region, China, Environmental Science & Technology Letters,doi: 10.1021/acs.estlett.0c00511

Shi, L., A. Zhu, L. Huang, E. Yaluk, Y. Gu, Y. Wang, S. Wang, A. Chan, and L. Li (2021), Impact of the planetary boundary layer on air quality simulations over the Yangtze River Delta region, China, Atmospheric Environment, 263, 118685, doi:10.1016/j.atmosenv.2021.118685.

Shu, L., T. Wang, H. Han, M. Xie, P. Chen, M. Li, and H. Wu (2020), Summertime ozone pollution in the Yangtze River Delta of eastern China during 2013–2017: Synoptic impacts and source apportionment, Environmental Pollution, 257, 113631, doi:10.1016/j.envpol.2019.113631.

Wang, H., et al. (2022), Seasonality and reduced nitric oxide titration dominated ozone increase during COVID-19 lockdown in eastern China, npj Climate and Atmospheric Science, 5(1), 24, doi:10.1038/s41612-022-00249-3.

Wang, X., et al. (2021), Modelling air quality during the EXPLORE-YRD campaign – Part I. Model performance evaluation and impacts of meteorological inputs and grid resolutions, Atmospheric Environment, 246, 118131, doi:10.1016/j.atmosenv.2020.118131.

Zhan, C., et al. (2020), Ozone affected by a succession of four landfall typhoons in the Yangtze River Delta, China: major processes and health impacts, Atmos. Chem. Phys., 20(22), 13781-13799, doi:10.5194/acp-20-13781-2020.